Oxygen

Physical Needs Team

CYP may require oxygen in an educational setting for a variety of reasons.

The main types of oxygen tanks used in settings:

- Compressed oxygen.

- Oxygen concentrator.

Oxygen Cylinders

- There are several different types of static and portable cylinders that look similar but deliver a range of flow rates.

- A ‘regulator’ is sometimes attached to a cylinder which enables lower oxygen flow rates to be delivered.

- Carry bags are available for portable cylinders.

- If a CYP is carrying their own portable cylinder a trolley can be provided by the supplier.

Oxygen concentrator

- Is a machine operated by electricity.

- Needs to be plugged into the main wall socket when being used - adaptors/ extension leads must not be used as they may reduce the power supply to the concentrator.

- It draws air from the room atmosphere, through the inlet, passing a filter that removes dust– The air is compressed before it passes through a chamber removing nitrogen but allowing the oxygen to pass through.

- Filters must be in place and totally dry before operating the unit.

- Oxygen is collected in a product tank where it is concentrated to a high purity before being pumped down the tubing at the prescribed flow rate.

- When concentrators are used, the setting must be supplied with an alternate source of oxygen, such as a spare oxygen cylinder.

- Ventilation parts must be unobstructed by any items which may impede ventilation.

- For safety reasons the concentrator should be switched off when not in use.

Oxygen

The oxygen provider will:

- Deliver oxygen as required and install oxygen equipment.

- Advise on the safest place for storage.

- Provide user guides for all oxygen equipment.

- Provide an emergency backup cylinder (if using an oxygen concentrator).

- Provide required consumables e.g. nasal cannula, oxygen tubing, face masks.

- Provide a carry bag for portable oxygen cylinders.

- Service the equipment at regular interval.

- Complete a risk assessment every six months.

Emergency Cylinders

- Emergency cylinders are provided in case of a power cut or machine failure.

- They need to be located where they can be easily accessible.

Staff Training

- Before a CYP can be administrated oxygen, relevant staff need to have completed training in the safe use of oxygen.

- If the CYP is known to the Community Nursing team/Outreach nursing team contact them to arrange training. Otherwise the school nurse or parents can deliver the training.

Oxygen

Health Care Plan

- Is written by clinical staff/school nurse in partnership with parents and the SENCO.

- Will identify medical conditions and what actions to follow to ensure the CYP can be safely supported.

- Will need to outline the emergency protocol.

- Staff working with the CYP must be familiar with the health care plan.

- Will need to be regularly reviewed at least annually or earlier if the CYP’s needs change

Who prescribes the oxygen?

- The CYP’s consultant. Nurses can amend this e.g. if the amount of oxygen needs to be increased.

Risk Assessment

- The oxygen provider will visit the setting ahead of the CYP starting in the setting and perform a field-based risk assessment before delivering oxygen equipment.

Swimming:

- Contact the health care provider to arrange extra tubing and nasal prongs.

- Inform staff at the swimming pool ahead of swimming.

- Inspect the nasal prongs after swimming to check if water is inside and replace if needed.

School trips/ Outings/ Residentials

- Ensure there is the correct amount of oxygen for the duration of the trip.

- Ensure there is a trained member of staff to accompany the CYP.

- Contact transport ahead of the trip to check what amount of oxygen they are permitted to carry and to ensure there is space to carry the oxygen.

- Alternate transportation may be required.

- The vehicle window will need to be kept partially open when oxygen equipment is in use.

Oxygen

Relevant people to notify when oxygen is on site

- The educational setting needs to inform their insurance.

- Fire Rescue Service: the presence and location of the oxygen cylinder storage areas.

- Electricity company: if using a concentrator- to be made aware of the CYP if the power supply is ever interrupted.

- Building manager (fire safety).

Safety:

- As oxygen is prescribed it is subject to medicine management protocol as well as safety precautions.

- Place statutory hazard notices in areas where oxygen is stored, including the classrooms.

- Ensure all relevant safety information is communicated to staff.

- Concentrators should be used in an open space near a plug and at least 1.5 meters away from a heat source and 3m away from naked flames e.g. Bunsen burner, candles, gas cooker.

Oxygen

- Oxygen cylinders can easily fall and be knocked over. They need to be stored appropriately and securely e.g. locked container with adequate ventilation and out of direct sunlight.

- Ensure the positioning of equipment does not cause a trip hazard.

- Be aware of where the tubing is at all times e.g. ensure it does not become tangled, kinked or become trapped or crushed under furniture as this can stop/ reduce the oxygen flow.

- Keep it away from feet when walking to prevent it becoming a trip hazard.

- The length of the tubing should only be altered by the supplier.

Statutory documents:

‘Supporting pupils at school with medical conditions’.

https://www.gov.uk/government/publications/supporting-pupils-at-school-with-medical-conditions--3

Useful contacts:

Community nursing team Bradford: 01274 365051

Children’s Outreach Team Airedale: 01535 293720

This leaflet gives general information.  For advice and support contact the relevant health professional.

Physical Needs Team